Node Voltage Analysis (NVA)

applicable to linear networks

NVA gives you a process for obtaining n equations in n unknowns.

The unknowns are the node voltages.

Once you have the node voltages, you can find any voltage or current anywhere in the circuit.

Process:

(Note the key words to remember each step.)

- Draw
Draw the circuit diagram indicating all element and source values.

- Label
Label a reference node, usually ground or a node with the most branches.
Then label a node voltage for all the other nodes in the circuit. 

You must recall the definition of a node voltage: 
It is the voltage of the node with respect to the reference node.  
It is not the same thing as the voltage across a branch. 
In general, the voltage across a branch between nodes 1 and 2will be the difference of the two node voltages on either side: 
 					v12 = v1 – v2

- Dependant
Write the controlling quantity of any dependant source in terms of node voltages. 
You will need to look for the controlling quantity in the circuit to do this.

- Super
Identify all the supernodes. 
Then write the voltage of the source inside the supernode in terms of the node voltages on either side.  

You must recall the definition of a supernode:
It is any voltage source with the two nodes on either side of it considered as one big node.

Note:  Any equations that you get in this step will be part of the n equations.

- Kirchoff 
Apply KCL to each node/supernode that is not part of the reference node.
These equations will supply the rest of the n equations.

Mesh Current Analysis (MCA)

applicable to linear planar networks

MCA gives you a process for obtaining n equations in n unknowns.

The unknowns are the mesh currents.

Once you have the mesh currents, you can find any voltage or current anywhere in the circuit.

Process:

(Note the key words to remember each step.)

- Draw
Draw the circuit diagram indicating all element and source values.

- Label
Label the mesh currents for all meshes that have a current source on the perimeter.
Then label a mesh current for all the other meshes in the circuit.

You must recall the definition of a mesh current:
It is the current that circulates inside a mesh.
It is not the same thing as a current through a branch.
In general, the current through a branch between meshes 1 and 2 will be the difference of the two mesh currents on either side: 
 					ibranch = i1 –  i2

- Dependant 
Write the controlling quantity of any dependant source in terms of mesh currents. 
You will need to look for the controlling quantity in the circuit to do this.

- Super
Identify all the supermeshs. 
Then write the current of the source inside the supermesh in terms of the mesh currents on either side. 

You must recall the definition of a supermesh:
It is any current source with the two meshes on either side of it considered as one big mesh.

Note:  Any equations that you get in this step will be part of the n equations.

- Kirchoff
Apply KVL to each mesh/supermesh that doesn’t have a current source on the perimeter.  
These equations will supply the rest of the n equations.