The Op-Amp as an Integrator

Consider an op-amp with infinite input resistance and zero output resistance:

- Ground the non-inverting input of the op-amp.

- Install a capacitor from the output to the inverting input.

- Connect a source vs to the inverting input through a resistor.

The result is an integrator:								 C

C

- +   vc  -

R									R

i

-

+			   +

_

- +		     +			vi

- +			     vo	       vs      _				      		   vo

+						     _

- vs							             					                +   AvI          _

_											_

- KVL around perimeter gives:	-vs  +   Ri +  1/C 0t i dt  + vc(0) + vo = 0

- and noting that			i =  (vs  - vi ) / R  	&	vi = - vo / A

- we get					(1 + 1/A) vo =  - 1/RC 0t (vs +  vo /A) dt   -  vc(0)

- As   A    we get			  vo =  - (1/RC) 0t vs dt  - vc(0)

So now we can see why this element is called an integrator!

The output voltage is proportional to the time integral of the input voltage