The Operational Amplifier

The operational Amplifier (Op-Amp) is essentially a voltage controlled dependent voltage source. The output voltage drawn across the output terminals depends on (or is controlled by) the control voltage which is at the input terminals

_

+

inverting					 +	     vi

input	  _					      -		  +		    +

v1						 v1	     +

vo

v2	    	  +					     v2

non-					            _	     _			 	     _

inverting input

Symbol Commonly Used				Symbol with Ground Shown

- vi = v1 – v2 = input voltage

- The output vo amplifies vi (on the order of 104 – 107) and reverses the polarity.

- If v2 = 0, then v1 appears amplified and inverted at the output.

- If v1 = 0, then v2 appears amplified and non-inverted at the output.

A useful model of the Op-Amp:

+				      +							  +

vi  			      -

-       		           Avi	 		  vo

v1		   +					    +

v2

-		  -									  -

Sometimes there is an input resistance Ri (104 - 107 ohms) between the two input terminals and an output resistance Ro (1-1000 ohms) in series with the dependent source and the output terminal.

The Op-Amp as a Voltage Follower (or Buffer Amp)

-

+

+

vs  					      vo

-

Note:

- vs = v2 = v1 = vo  The output voltage follows the input!

- the loop current must be zero

A model of the circuit:

vi

v1					    vo

vs			       v2				  Avi

KVL:    vs + Avi  + vi  =  0            Avi  + vi  =  - vs          (A  + 1) vi  = -vs           vi  =	  - vs

(A  + 1)	    vo = - Avi

vo =      A     vs             vo   vs   “The output follows the input”

(A+1)

This “unity” gain amplifier has the advantages that although the input draws negligible current and power from the source, the output can supply reasonable current (10 – 20 mA) and power (100 – 500 mW) to a load connected across the output.

Hence the load has little effect on the source and that is why it is also called a Buffer Amplifier.