An Inductor is an energy storage lement.

It is a two-terminal circuit element that is a model of a real device that consists of

a coil of resistance-less wire wound around a material.

i	          +   v   -

L

Inductance

L is the inductance of the device.

L = (oN2A) /(l + 0.45d)  	(if l >d/2 & core is non-ferromagnetic)

where

- N is the number of turns of the wire

- A is the cross sectional area of the loops

- l is the length of the coil

- d is the diameter of the wire

- o = the permeability of free space = 4 x 10-7 H/m. 
In this case the material inside the coil is a vacuum.

Inductance is a property of the device that measures the ability of the device to store energy in the form of a magnetic field.

The units of inductance are Volt-Seconds per Ampere or Henrys.

The voltage-current characteristic for an inductor is:

v = L  di/dt

-or-

i = 1/ L  tot  v dt + i(to)

Two observations from the v-i characteristic:

- There is no voltage across an inductor if the current through it is not changing with time.
That is why we say that “inductors act as a short circuit to a dc current”.

- Since there can not be infinite voltages, the current through an inductor cannot change instantaneously. (However, the voltage can.) This means that iL(t+) = iL(t-)

The energy stored in an inductor is:

(wL)0-->t   =   wL(t)  = ½ L i2  –   ½ L i02

From this we see that the energy stored in an inductor is only a function of the current through it.

So even if the voltage is 0, there can be some energy in an inductor.

An inductor is a passive element.

From the expression for energy stored in an inductor we see that (wL)-inf-> t is always > 0.

Equivalent Inductors, Leq

Combinations of inductors can be reduced to an equivalent inductance.

They combine like resistors.

Practical Model of an Inductor

A more practical model of an inductor is an ideal inductor in series with a small resistance.

A Capacitor is an energy storage element.

It is a two-terminal circuit element that is a model of a real device that consists of

two parallel conducting plates separated by a non-conducting material.

i	          +  v  -

C

Capacitance

C is the capacitance of the device.

C =  A / d

where

- A is area of the plates

- d is the distance between the plates

-  is the dielectric constant, a property of the material between the plates 
The dielectric constant is a property of a material that is a measure of the materials ability to store energy per unit volume for unit voltage difference.

Capacitance is a property of a device that measures the ability of the device to store energy in the form of an electric field.

The units are Coulombs per Volt or Farads.

The voltage-current characteristic for a capacitor is:

i = C  dv/dt

-or-

v = 1/ C tot  i dt + v(to)

Two observations from the v-i characteristic:

- There is no current through a capacitor if the voltage across it is not changing with time.
That is why we say that “capacitors act as an open circuit to a dc voltage”.

- Since there can not be infinite currents, the voltage across a capacitor cannot change instantaneously. (However, the current can.) This means that vC(t+) = vC(t-)

The energy stored in a capacitor is:

(wC)0-->t  =   wC(t)  = ½ C v2  –   ½ C v02

From this we see that the energy stored in a capacitor is only a function of the voltage across it.

So even if the current is 0, there can be some energy in a capacitor.

A capacitor is a passive element.

From the expression for energy stored in a capacitor we see that (wC)-inf-> t is always > 0.

Equivalent Capacitors, Ceq

Combinations of capacitors can be reduced to an equivalent capacitance.

They combine like conductors.

Practical Model of a Capacitor

A more practical model of a capacitor is an ideal capacitor in parallel with a large resistance.

A note about Linearity:

The voltage-current relationships for capacitors and inductors are linear.

Hence these will hold for circuits with inductors and capacitors:

- KVL

- KCL

- Thevenin’s Theorem

- Norton’s Theorem

- Principal of superposition