Plumbing  a Hometime Video

Different Projects in a House

Understanding Plumbing Systems-

Plumbing projects in a new house under construction happen during the rough-in stage.

3 basic systems:

Bring the water into the house (supply- copper pipes)

Drain the waste out of the house and vent the air out (PVC)

This house will have common fixtures—a tub, shower, vanities, sinks, and a whirlpool.

A soil stack is the pipe system that ties into the main house drain or municipal sewage system and can also serve as a vent that draws air in.

Plumbing often requires permits and inspection and is closely regulated.  Codes require each fixture to have a trap.

Installing DWV Lines-

Begin with demolition and dismantling the old system.

Take the soil stack apart with a bit maul and remove, so that it can be moved nearer foundation posts.  Cut it near the floor with a soil pipe snapper. Plug it up to prevent gases from entering the house.

Mark and cut the openings in the walls and framing for fixtures and drains.  Outline the hole for the toilet flange and cut.

For the shower drain, center the hole, mark it , and cut. Put in the flange, stringer, and plug it up.

Be sure to cut the holes to the outside diameter.

Use ABS pipe.  It’s rigid, so cut with a miter saw or hacksaw.  Use ABS glue to connect. It sets quickly.

Secure a straight T for tub and toilet junction and a reducing T for drains.

Tub vent-  Drill through joists.

Check for a fall of 1/4” for every foot of run.  Nail in braces to code and under Ts. Saw a 4” hole in line but progressively lower,

A reducing Y ties in shower drain.

A cufflink connects 3” ABS to 4” soil stack.  Connects with a sanitary reducing T.

All fixtures need vents to decrease noise and increase flow.

For the master bath whirlpool, don’t cut into joists.

Double vanity sinks can use a single drain.  A cross ties them together with a cleanout drain and a vent above the drain.

## Installing Water Supply Lines

Use 1/2’ and 1/4” copper pipe.

Start on highest floor and work down.

If possible, assemble the pieces on the floor first.

Hot is on the right and cold is on the left.

Sweating the joints—melting the solder.

- Clean the pipe with emery cloth

- Use flux to prevent oxidation.

- Heat with a propane torch.

Wrap Teflon tape around threads.  Secure shower arm with a fitting.  Nail on guards so no one can puncture the pipe with nails or drywall screws.

Move in the tub during the rough-in stage.

Use reducing Ts for branches.  Use up to 3 fixtures on every 1/2” branch.

Sometimes it’s useful to assemble first and sweat the joints later.

In the basement, reducing Ts connect vanity and tie into the main water supply.

Valves for every fixture are useful to shut off any part of the system without having to turn off the main valve for the whole house.

Installing Shower Fixtures

## Installing a New Toilet

Installing Vanity Fixtures

Installing Whirlpool Tub

Installing Sink, Dishwasher, and Garbage Disposal.