Practical Voltage Sources

- An ideal voltage source is a device whose terminal voltage is independent of the current through it.

- It does not actually exist.

(Although an OK approximation for small ranges of current or power drawn from it.)

- To approximate a real device we must account for lowering of the voltage when there is a large current drawn from it.

- So we model a practical voltage source as an ideal one in series with a resister.

RL= 0, vL = 0

Rsv					   iL	       iLsc= vs/ Rsv	
    iL						
						  RL= Rsv

+						vL = ½ vs	   ideal source

vs					vL

RL

practical		   RL= , iL = 0

source		               vLoc = vs

vL

vs = voltage of the ideal source

Rsv = internal resistance or output resistance

vL = load voltage or terminal voltage

iL  = load current

RL = load resistor

iLsc = short circuit current

vLoc = open circuit voltage

Use KVL to get:	 	vs = iL Rsv + vL

iL = (vs/ Rsv) – (vL/ Rsv)

This is the equation of the line for the practical voltage source.

# Each point on the line corresponds to a different value of RL.

For this practical voltage source, the terminal voltage is near that of an ideal one only for small values of load current, (obtained with values of RL  that are large compared with Rsv.)

Practical Current Sources

- An ideal current source is a device that will deliver a constant current regardless of the voltage across it.

- It does not actually exist.  
(Although an OK approximation for small ranges of voltage or load.)

- To approximate a real device we must account for lowering of the current when there is a large load resistance.

- So we model a practical current source as an ideal one in parallel with a resister.

RL= 0, vL = 0

iL	     iLsc = is	       ideal source
iL

+							  RL= Rsi

is					vL				  			  iL = ½ is

Rsi		     RL			       practical

source			  RL= , iL = 0

vLoc = Rsi is

vL

is = current of the ideal source

Rsi = internal resistance or output resistance

Use KCL to get:		is = iL + (vL/ Rsi)

iL = is – (vL/ Rsi)

This is the equation of the line for the practical current source.

Each point on the line corresponds to a different value of RL.

Note:  From previous page we got the equation for a practical voltage source: iL = (vs/ Rsv) – (vL/ Rsv)

These equations are the same if:   is = (vs/ Rsv)  and Rsi = Rsv    (=R)

So a source transformation can be accomplished by replacing a practical cs with a practical vs (or vice-versa) and giving the appropriate values for is , vs , and R.

# For this practical current source, the load current is equal to the source current only for small values of the load voltage vL, (obtained with values of  RL that are small compared with Rsi.)

Source Transformations

### Two sources are equivalent if they produce identical values of vL and iL when connected to identical values of  RL, no matter what RL may be.

### So a practical voltage source and a practical current source can be equivalent if:

vLoc = vs = Rsi is	Rsi  = Rsv = Rs = the internal resistance of either practical source

iLsc = vs/ Rsv  = is	vs = Rs is

2

+

3A			   2			 6V    _

Note:  Just because their voltage-current characteristics are the same doesn’t mean that they will deliver the same power.

If there is a 4 load, iL = 1A, vL = 4V and power absorbed is 4W for both.

However, the current source delivers 12 W and Rs absorbs 8W, while the voltage source delivers 6W and Rs absorbs 2W.

So they are not the same internally!

Maximum Power Transfer Theorem

## An independent voltage source in series with a resistance Rs,

## or an independent current source in parallel with a resistance Rs ,

## delivers a maximum power to RL, when Rs = RL.

## It delivers 0 power when Rs = 0 or when RL = .

## Note:  pL = iL2 RL = [vs / (Rs + RL)] 2 RL = [(vs 2 RL ) / (Rs + RL)2]

To get maximum power take the derivative wrt RL and set it = 0.  You will find that Rs = RL